| Protocol No |  |  | Sample Issued to |  | Sample Issued by |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sample Name: |  |  |  | Sample Received On: |  |  |  |
| Lab Code No |  |  |  | Date of Analysis: |  |  |  |

Result Table:

| Sr. No. | Parameter | Method | Result | Unit |
| --- | --- | --- | --- | --- |
| 1 | pH (at 25oC) | IS 3025 : Part 11 |  |  |
| 2 | Total Dissolved Solids (TDS) | IS 3025 : Part 16 |  | mg/L |
| 3 | Chlorides as Cl | IS 3025 : Part 32 |  | mg/L |
| 4 | Total Alkalinity as CaCO3 | IS 3025 : Part 23 |  | mg/L |
| 5 | Conductivity at 25oC | IS 3025 : Part 14 |  | μs/cm |
| 6 | Total Hardness as CaCO3 | IS 3025 : Part 21 |  | mg/L |
| 7 | Calcium as Ca | IS 3025 : Part 40 |  | mg/L |
| 8 | Calcium as CaCO3 | IS 3025 : Part 40 |  | mg/L |
| 9 | Magnesium as Mg | IS 3025 : Part 46 |  | mg/L |
| 10 | Odor | IS 3025 : Part 5 |  |  |
| 11 | Turbidity | IS 3025 : Part 10 |  | NTU |

Analyzed By:                                                                                                                               Checked By:

Name                                                                                                                                            Technical Manager:        Dated signature:                                                                                                                        Dated Signature:

| Protocol No |  |  | Sample Issued to |  | Sample Issued by |  |
| --- | --- | --- | --- | --- | --- | --- |

OBSERVATION TABLE:

- APPEARANCE:

| Sr. No | Parameter | Reading | Unit |
| --- | --- | --- | --- |
| 1 | Odour |  |  |
| 2 | Turbidity |  | NTU |
| 3 | pH at 25oC |  |  |
| 4 | Conductivity at 25oC |  | μs/cm |

- Total Dissolved Solids (TDS):

| Sr.No | Description | Readings |
| --- | --- | --- |
| 1 | Volume of sample taken (W) |  |
| 2 | Weight of Empty Dish (W1) |  |
| 3 | Weight of Dissolved Solids + Dish (W2) |  |
| W2-W1 x1000x1000 |  |  |
| TDS= --------------------------------- W |  | mg/L |

Analyzed By:                                                                                                                          Checked By:

Name :                                                                                                                                      Technical Manager:

Dated signature :                                                                                                                    Dated signature :

| Protocol No |  |  | Sample Issued to |  | Sample Issued by |  |
| --- | --- | --- | --- | --- | --- | --- |

- Chlorides as Cl:

| Sr.No | Description | Readings |
| --- | --- | --- |
| 1 | Volume of sample taken (V3) |  |
| 2 | Burette reading (B.R) of sample ( V1) Burette reading (B.R) of Blank ( V2) |  |
| 3 | Standardization of AgNO3 Normality of AgNO3 (N) = |  |
|  | Calculation: (V1-V2) X N X 35.45 X 1000 |  |
|  | Chloride mg/L= --------------------------------- = V3 | mg/L |

- Alkalinity as CaCO3:

| Sr.No | Description | Readings |
| --- | --- | --- |
| 1 | Volume of sample taken (V) |  |
| 2 | Burette reading (B.R) (A) |  |
| 3 | Standardization of sulphuric acid Normality of sulphuric acid (0.02N) ( N) |  |
|  | Calculation: A X N X 50000 |  |
|  | Alkalinity (mg/L) as CaCO3 = ------------------------ V | mg/L |

Analyzed By:                                                                                                                          Checked By:

Name :                                                                                                                                      Technical Manager:

Dated signature :                                                                                                                    Dated signature :

| Protocol No |  |  | Sample Issued to |  | Sample Issued by |  |
| --- | --- | --- | --- | --- | --- | --- |

- Magnesium as Mg:

| Sr.No | Description | Readings |
| --- | --- | --- |
| 1 | Calculation: Mg(mg/L) = (Total Hardness as CaCO3 – Calcium hardness as CaCO3) X 0.243 | mg/L |

- Total Hardness as CaCO3

| Sr.No | Description | Readings |
| --- | --- | --- |
| 1 | Volume of sample taken |  |
| 2 | Burette reading (B.R) (A) |  |
| 3 | Standardization of EDTA 1ml 0.01 M = mg of CaCO3 (B) |  |
|  | Calculation: A X B X 1000 |  |
|  | Hardness (mg/L) as CaCO3 = ------------------------ ml of sample | mg/L |

Analyzed By:                                                                                                                          Checked By:

Name :                                                                                                                                      Technical Manager:

Dated signature :                                                                                                                    Dated signature :

| Protocol No |  |  | Sample Issued to |  | Sample Issued by |  |
| --- | --- | --- | --- | --- | --- | --- |

- Calcium as Ca :

| Sr.No | Description | Readings |
| --- | --- | --- |
| 1 | Volume of sample taken |  |
| 2 | Burette reading (B.R) (A) |  |
| 3 | Standardization of EDTA 1ml 0.01 M = mg of Ca ( B) 1ml 0.01 M = mg of CaCO3 (C) |  |
|  | Calculation: A X B X 1000 |  |
|  | Calcium (mg/L) as Ca = -------------------- ml of sample | mg/L |
|  | (B.R)xC×1000 |  |
|  | Calcium (mg/L) as CaCO3= ------------------- ml of sample | mg/L |

Analyzed By:                                                                                                                          Checked By:

Name :                                                                                                                                      Technical Manager:

Dated signature :                                                                                                                    Dated signature :