ULR No: TC161182600063801F                                                                                                 QSF No. 7.8.2

TEST REPORT

Date: 30/07/2026

| Customer Name & Address |  |  |  |
| --- | --- | --- | --- |
| Customer Sample ID | Drinking Water | Batch No. | --- |
| Lab Code | SLS/26/638/01 | Date of Sampling | --- |
| Date of Sample Receipt | 27/07/2026 | Location of sampling | --- |
| Sample Name | Potable Water submitted as Drinking Water |  |  |
| Sampling Method | --- | Sampling Done by | --- |
| Condition Of Sample | Packed in plastic container | Test Performance Date | 27/07/26 – 30/07/26 |
| Sample Appearance | Colorless transparent liquid | Sample quantity | 1 lit |
| Testing conducted at | Temperature – 250C ±20C, Humidity – 55 % ±10 % |  |  |

Report No:  SLS/26/638/01/01

CHEMICAL TEST REPORT

| Sr. No | Name of Test | Result | Limit for Drinking Water as per IS 10500:2018 Standards |  | Method of Analysis |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Desirable Limit | Permissible limit |  |  |
| Chemical Test |  |  |  |  |  |  |
| 1 | pH at 250C | 7.320 | 6.5 - 8.5 | 6.5 - 8.5 | IS 3025 (P-11) |  |
| 2 | Total Dissolved Solids (TDS) | 86.00 mg/l | Max.500 mg/l | Max.2000 mg/l | IS 3025 (P-16) |  |
| 3 | Chlorides as Cl | 19.71 mg/l | Max.250 mg/l | Max.1000 mg/l | IS 3025 (P-32) |  |
| 4 | Total Alkalinity as CaCO3 | 52.00 mg/l | Max.200 mg/l | Max.600 mg/l | IS 3025 (P-23) |  |
| 5 | Conductivity at 250C | 139.5 µs/cm | -- | -- | IS 3025 (P-14) |  |
| 6 | Total Hardness as CaCO3 | 08.51 mg/l | Max.200 mg/l | Max.600 mg/l | IS 3025 (P-21) |  |
| Elemental Test |  |  |  |  |  |  |
| 7 | Calcium as Ca | 31.83 mg/l | Max.75 mg/l | Max.200 mg/l | IS 3025 (P-40) |  |
| 8 | Magnesium as Mg | 0.19 mg/l | Max.30.0 mg/l | Max.100 mg/l | IS 3025 (P-46) |  |
| Remark: The sample analyzed confirms as per IS 10500:2018 Standards with respect to above tests performed. The Results are pertaining to the sample sent for analysis only. |  |  |  |  |  |  |

Mrs. Amruta Kulkarni	                                                                                            Checked by:

Quality Manager

Authorized Signatory

For SLS

Disclaimer

- Sample submitted by the customer in their own container.

- Above analysis result is valid only for specific sample as stated above, without any bias to its source.

- We claim no responsibility for changes made in the report after dispatch e.g. Use of whitener or eraser.

- Above test report cannot be produced as legal evidence without our prior written permission.

- Sample stored for one week and test Report for one year from the date of report dispatch.

Duplicate copies of Report or Invoice will be charged extra.

End of Reports

Page 1 of 1

QSF No. 7.8.2

TEST REPORT

Date: 30/07/2026				                       Report No: SLS/26/638/01/02

| Customer Name & Address |  |  |  |
| --- | --- | --- | --- |
| Customer Sample ID | Drinking Water | Batch No. | --- |
| Lab Code | SLS/26/638/01 | Date of Sampling | --- |
| Date of Sample Receipt | 27/07/2026 | Location of sampling | --- |
| Sample Name | Potable Water submitted as Drinking Water |  |  |
| Sampling Method | --- | Sampling Done by | --- |
| Condition Of Sample | Packed in plastic container | Test Performance Date | 27/07/26 – 30/07/26 |
| Sample Appearance | Colorless transparent liquid | Sample quantity | 1 lit |
| Testing conducted at | Temperature – 250C ±20C, Humidity – 55 % ±10 % |  |  |

CHEMICAL TEST REPORT

| Sr. No | Name of Test | Result | Limit for Drinking Water as per IS 10500:2018 Standards |  | Method of Analysis |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Desirable Limit | Permissible limit |  |
| Physical Test |  |  |  |  |  |
| 1 | Odor | Agreeable | Agreeable | Agreeable | IS 3025 (P-5) |
| 2 | Turbidity | <1 NTU | Max.1 NTU | Max.5 NTU | IS 3025 (P-10) |

MICROBIOLOGICAL TEST REPORT

| Microbiological Test |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sr.No | Name of Test | Result | Limit for Drinking Water as per IS 10500:2018 Standards |  | Method of Analysis |
| 4 | T. Coli form Bacteria | Absent/100ml | Absent/100ml |  | IS 1622 |
| 5 | Eschericia coli | Absent/100ml | Absent/100ml |  | IS 1622 |
| Remark: The sample analyzed confirm as per IS 10500:2018 Standards with respect to the above tests performed. The Results are pertaining to the sample sent for analysis only |  |  |  |  |  |

Mrs. Amruta Kulkarni	                                                                                            Checked by:

Quality Manager

Authorized Signatory

For SLS

Disclaimer

- Sample submitted by the customer in their own container.

- Above analysis result is valid only for specific sample as stated above, without any bias to its source.

- We claim no responsibility for changes made in the report after dispatch e.g. Use of whitener or eraser.

- Above test report cannot be produced as legal evidence without our prior written permission.

- Sample stored for one week and test Report for one year from the date of report dispatch.

- Duplicate copies of Report or Invoice will be charged extra.

End of Reports                                                                                                                                Page 1 of 1